FULL HD PTZ CAMERA

User Manual

（V1.0）

- Attentions

This manual introduces functions, installations and operations for this PTZ camera in details. Please read this manual carefully before installation and use.

1. Cautions

1.1 Avoid damage to product caused by heavy pressure, strong vibration or immersion during transportation, storage and installation.

1.2 Housing of this product is made of organic materials. Do not expose it to any liquid, gas or solids which may corrode the shell.

1.3 Do not expose the product to rain or moisture.

1.4 To prevent the risk of electric shock, do not open the case. Installation and maintenance should only be carried out by qualified technicians.

1.5 Do not use the product beyond the specified temperature, humidity or power supply specifications.

1.6 Wipe it with a soft, dry cloth when cleaning the camera lens. Wipe it gently with a mild detergent if needed. Do not use strong or corrosive detergents to avoid scratching the lens and affecting the image;

1.7 This product contains no parts which can be maintained by users themselves. Any damage caused by dismantling the product by user without permission is not covered by warranty.

2. Electrical Safety

Installation and use of this product must strictly comply with local electrical safety standards.

The power supply of the product is ±12V, the max electrical current is 2A.

3. Install

  - Do not rotate the camera head violently, otherwise it may cause mechanical failure.

  - This product should be placed on a stable desktop or other horizontal surface. Do not install the product obliquely, otherwise it may display inclined image;

  - Ensure there are no obstacles within rotation range of the holder.

  - Do not power on before completely installation.

4. Magnetic Interference

Electromagnetic fields at specific frequencies may affect the video image. This product is Class A. It may cause radio interference in household application. Appropriate measure is required.

Content

1. Camera Installation	3

1.1.Camera Introduction	3

1.2 Power-on Initial Configuration	3

1.3 Video Output	4

1.4 Mounting Brackets	4

2. Product Overview	6

2.1 Product Introduction	6

2.1.1 Model	6

2.1.2 Dimension	6

2.1.3 Accessory	7

2.2 Main Features	7

2.3 Technical Parameter	8

2.4 Interface Instruction	9

2.4.1 External Interface	9

2.4.2 RS-232 Interface	10

3. Application Instruction	12

3.1 Video Output	12

3.1.1 Power-On Initial Configuration	12

3.1.2 Video Output	12

3.2 Remote Control	13

3.2.1 Keys Introduction	13

3.3 Menu Introduction	16

3.3.1 Main Menu	16

3.3.2 System Setting	16

3.3.3 Camera Setting	17

3.3.4 P/T/Z	20

3.3.5 Version	21

4. Serial Port Communication and Control	22

4.1 VISCA Protocol list	22

4.1.1 VISCA Protocol Return Command	22

4.1.2 VISCA Protocol Control Command	22

4.1.3 VISCA Protocol Inquiry Command	25

4.2 Pelco-D Protocol Command List	27

4.3 Pelco-P Protocol Command List	27

5. Maintenance and Troubleshooting	28

5.1 Camera Maintenance	28

5.2 Troubleshooting	28

6. Copyright Statement	29

# 1. Camera Installation

## Camera Introduction

| Figure 1.1 Product Interfaces |  |  |
| --- | --- | --- |
| 1. Camera Lens | 5. Screw Hole for Tripod | 9. USB 2.0 Interface |
| 2. Remote Control Receiving Indicator | 6. RS232 Control Interface（Input） | 10. DC12V Input Power Supply Socket |
| 3. Camera Base | 7. RS232 Control Interface（Output） |  |
| 4. Tripod Screw Hole | 8. RS485 Input (left +, right-) |  |

## 1.2 Power-on Initial Configuration

1) Power-on: Connect DC12V power supply adapter with power supply socket

2) Initial Configuration: The remote control receiving indicator flashes after power on, the pan-tilt turns left to the lowest left to the lowest, and then turns to the HOME position (both the horizontal and vertical positions are in the middle), while the movement first shrinks and then stretches. When remote control receiving indicator stops flashing, the self-checking is completed.

Notes:

- After power on and self-checking, the camera will automatically return to the preset 0 position if it’s pre-set.

- The default address for the IR remote control is 1#. If the menu restored to factory defaults, the remote control default address will restore to 1#.

## 1.3 Video Output

USB2.0 Video Output

a. USB 2.0 video cable connection: as shown in Figure1.1 label 9

b. Connect the camera and the computer via USB2.0 video cable, open the video software and select the image device to output the video.

## 1.4 Mounting Brackets

Notes: Ceiling or wall mounting brackets can only be mounted on template and concrete wall.

For safety reason, plasterboard is not recommended.

- Wall Mounting:

- Ceiling Mounting

2. Product Overview

## 2.1 Product Introduction

### 2.1.1 Model

Model No.X — XXX

10---10X Optical Zoom Lens

Figure 2.1 Product Model

### 2.1.2 Dimension

Figure 2.2 Product Dimension

### 2.1.3 Accessory

Please check below standard and optional accessories when unpacking the box.

| Standard Accessory | Optional Accessory |
| --- | --- |
| Power adapter | Wall Mount |
| IR Remote Control | Ceiling Mount |
| RS232 Cable |  |
| User Manual |  |
| USB2.0 Cable |  |

USB2.0 video cable: If USB2.0 video cable is required for power supply without power adapter, in which power is supplied by red end and USB video signal is transmitted by black end; if power adapter is used, USB2.0 video cable is not required for power supply, and ordinary USB2.0 video cable is enough.

## 2.2 Main Features

This series camera has perfect functions, superior performance and rich video output interfaces; Featuring with advanced ISP processing algorithms, offering vivid and high resolution video with a strong sense of depth and fantastic color rendition.

- Full HD Resolution: 1/2.9 inch high quality CMOS sensor. Resolution is up to 1920x1080 with frame rate up to 30 fps.

- Multiple Optical Zoom Lens: 3X/10X optical zoom lens.

- Leading Auto Focus Technology: Fast, accurate and stable auto focusing technology.

- Low Noise and High SNR: Super high SNR image is achieved with low noise CMOS. Advanced 2D/3D noise reduction technology further reduces the noise while ensuring high image clarity.

- Control Interface: RS485, RS232 (cascade connection)

- Multiple Control Protocol: Support VISCA, PELCO-D, PELCO-P protocols; Support automatic identification protocols.

- Quiet Pan / Tilt Movement: With high accuracy step driving motor, camera can pan / tilt extremely quiet and smooth.

- Multiple presets: Up to 255 presets (10 presets via remote control).

- Multiple Application: Online-education, Lecture Capture, Webcasting, Video conferencing, Tele-medicine, Unified Communication, Emergency command and control systems, etc.

## 2.3 Technical Parameter

| Model | 10X |
| --- | --- |
| Camera Parameter |  |
| Sensor | 1/2.9 inch high quality CMOS sensor |
| Effective Pixels | 2.07 megapixel, 16：9 |
| Video Format | H264/MJPG:1920*1080P@30/25/20/15/10/5fps；1280*720P@30/25/20/15/10/5fps；960*540@30/25/20/15/10/5fps；800*600@30/25/20/15/10/5fps；720*576@30/25/20/15/10/5fps；720*480@30/25/20/15/10/5fps；640*480@30/25/20/15/10/5fps；640*360@30/25/20/15/10/5fps；352*288@30/25/20/15/10/5fps；320*240@30/25/20/15/10/5fps； YUY2:1920*1080P@5fps；1280*720@10/5fps；1024*576@15/10/5fps；800*600@20/15/10/5fps；800*448@25/20/15/10/5fps；640*480@30/25/20/15/10/5fps；640*360@30/25/20/15/10/5fps；480*270@30/25/20/15/10/5fps；320*180@30/25/20/15/10/5fps； NV12:1280*720@10/5fps；1024*576@15/10/5fps；800*600@20/15/10/5fps；800*448@25/20/15/10/5fps；640*480@30/25/20/15/10/5fps；640*360@30/25/20/15/10/5fps；480*270@30/25/20/15/10/5fps；320*180@30/25/20/15/10/5fps； |
| View Angle | 8.8°～ 68.8° |
| Focus Length | f=4.34mm～41.66mm |
| AV | F1.85 – F2.43 |
| Optical Zoom | 10X |
| Minimum Illumination | 0.5Lux(F1.8, AGC ON) |
| DNR | 2D﹠3D DNR |
| White Balance | Auto / Manual/ One Push/Specified Color Temperature(2400K-7100K) |
| Focus | Auto / Manual/ One Push Focus |
| Exposure | Auto / Manual |
| BLC | On/Off |
| Video Adjustment | Brightness, Color, Saturation, Contrast, Sharpness, B/W mode, Gamma curve |
| SNR | >50dB |

| Input/output Interface |  |
| --- | --- |
| Video Output | USB2.0 Interface, A Type Interface |
| Video Compression Format | YUY2/NV12/MJPG/H.264 |
| Control Interface | RS232 (IN/OUT), RS485 |
| Control Protocol | VISCA/Pelco-D/Pelco-P |
| Power Interface | HEC3800 outlet (DC12V) |

| USB Feature |  |
| --- | --- |
| Operation Systems | Windows 7, Windows8, Windows10, Mac osx, Linux, etc |
| Video Compression Format | MJPG/H264/H265 |
| USB Communication Protocol | UVC |

| PTZ Parameter |  |
| --- | --- |
| Pan Rotation | -170°～+170° |
| Tilt Rotation | -30°～+30° |
| Pan Control Speed | 0.1 ～60°/sec |
| Tilt Control Speed | 0.1～40°/sec |
| Preset Speed | Pan: 48°/sec, Tilt: 38°/sec |
| Preset Number | 255 presets (10 presets via remote control) |

| Other Parameter |  |
| --- | --- |
| Input Voltage | 12V |
| Operating Current | Maximum:0.42A |
| Power Consumption | Maximum: 5W |
| Stored Temperature | -40℃～+70℃ |
| Storage Humidity | 20%～90% |
| Working Temperature | -10℃～+50℃ |
| Working Humidity | 20%～80% |
| Dimension (W*H*D) | 156.8mm×112.6mm×139.5mm |
| Weight | 1KG |
| Application | Indoor |
| Package | Power Supply, RS232 Control Cable, IR Remote Control, User Manual, Warranty Card, USB2.0 Cable |
| Optional Accessories | Ceiling / Wall Mount (Extra Cost) |

## 2.4 Interface Instruction

### 2.4.1 External Interface

Figure2.4 Wiring Diagram

### 2.4.2 RS-232 Interface

1）RS-232 Interface Definition

|  | Connection to PC or Camera Controller |  |
| --- | --- | --- |
|  | Camera | WindowsDB-9 |
|  | 1.DTR | 1.DCD |
|  | 2.DSR | 2.RXD |
|  | 3.TXD | 3.TXD |
|  | 4.GND | 4.DTR |
|  | 5.RXD | 5.GND |
|  | 6.GND | 6.DSR |
|  | 7.IR OUT | 7.RTS |
|  | 8.NC | 8.CTS |
|  |  | 9.RI |

2）RS-232 Mini-DIN 8-pin Port Definition

|  | NO. | Port | Definition |
| --- | --- | --- | --- |
|  | 1 | DTR | Data Terminal Ready |
|  | 2 | DSR | Data Set Ready |
|  | 3 | TXD | Transmit Data |
|  | 4 | GND | Signal Ground |
|  | 5 | RXD | Receive Data |
|  | 6 | GND | Signal Ground |
|  | 7 | IR OUT | IR Commander Signal |
|  | 8 | NC | No Connection |

3）RS232(DB9) Port Definition

|  | NO. | Port | Definition |
| --- | --- | --- | --- |
|  | 1 | DCD | Data Carrier Detect |
|  | 2 | RXD | Receive Data |
|  | 3 | TXD | Transmit Data |
|  | 4 | DTR | Data Terminal Ready |
|  | 5 | GND | System Ground |
|  | 6 | DSR | Data Set Ready |
|  | 7 | RTS | Request to Send |
|  | 8 | CTS | Clear to Send |
|  | 9 | RI | Ring Indicator |

4）VISCA networking as shown below

|  | Camera cascade connection |  |
| --- | --- | --- |
|  | Camera 1 | Camera 2 |
|  | 1.DTR | 1.DTR |
|  | 2.DSR | 2.DSR |
|  | 3.TXD | 3.TXD |
|  | 4.GND | 4.GND |
|  | 5.RXD | 5.RXD |
|  | 6.GND | 6.GND |
|  | 7.IR OUT | 7.OPEN |
|  | 8. NC | 8.OPEN |

3. Application Instruction

## 3.1 Video Output

### 3.1.1 Power-On Initial Configuration

The remote control receiving indicator flashes after power on, the pan-tilt turns left to the lowest left to the lowest, and then turns to the HOME position (both the horizontal and vertical positions are in the middle), while the movement first shrinks and then stretches. When remote control receiving indicator stops flashing, the self-checking is completed. After power on and self-checking, the camera will automatically return to the preset 0 position if it’s pre-set.

### 3.1.2 Video Output

Connect to the video output cable: the user can refer to Figure 1.1 product interfaces.

USB2.0 output: Connect the camera with the computer USB2.0 interface (black), open the Device Manager to see whether there is an imaging device and whether the Universal Serial Bus controllers recognize the USB2.0 device. After properly identified, open the software, choose the imaging device, and then it will output the image.

## 3.2 Remote Control

### 3.2.1 Keys Introduction

|  | 1) In this manual, “press the key” means a click rather than a long-press, and a special note will be given if a long-press for more than one second is required. 2) When a key-combination is required, do it in sequence. For example,“ 【*】+【#】+【F1】”means press“【*】”first and then press“【#】” and last press“【F1】”. 1. Standby Key The camera enters standby mode if long press 3s on standby key; Long press 3s again on the standby key, the camera will self-check again and return to HOME position (If preset 0 position is set, the camera will return to preset 0 position). 2. Camera Selection Select the camera address to control .. 3. Number Keys Set or call 0-9 number keys. - * and # Keys - Focus Control Keys: Adjust focus - [AUTO]: Enter into auto focus mode - [MANUAL]: Enter into manual focus mode. Press + or - to adjust focus - Zoom Control ZOOM +: press【ZOOM +】key to zoom in ZOOM - : press 【ZOOM -】key to zoom out - Set and Clear Presets Set Preset: press 【SET PRESET】 button, and then press the number key 0-9 to set preset positions. Clear Preset: press 【CLEAR PRESET】 button, and then press the number key 0-9 to clear preset positions. - Pan/Tilt Control Up: press Down: press Left: press Right: press “【HOME】”: Pan/Tilt back to the middle or enter into the next menu - BLC Key - BLC on/off: Turn on/off BLC - Menu Keys - Enter into/Exit OSD Menu or back to previous level menu |
| --- | --- |
|  | - Camera Remote Control Address Setting 【*】+【＃】+【F1】:Camera Address No.1 【*】+【＃】+【F2】:Camera Address No. 2 【*】+【＃】+【F3】:Camera Address No. 3 - 【*】+【＃】+【F4】:Camera Address No. 4 |

### 3.2.2 Remote Usage

After the device starts normally, it receives and executes the infrared command, presses the button of the remote control, the remote control receiving indicator flashes, releases the button, the indicator stops flashing. You can use the infrared remote control to perform operations such as preset position setting, positioning, pan/tilt control.

Key Description:

1. The button mode mentioned in this manual refers to the two actions of pressing and releasing the buttons on the remote control. For example, "press the [HOME] key" refers to the action of pressing the [HOME] key and then release. If you need to press the key for a long time, it will be specially explained in the manual.

2. In this manual, when it is necessary to operate a combination key, it means to operate in the order of the manual. For example, "press【*】+【#】+【F1】key" means to press the【*】key first, then press the【#】key, and finally press the【F1】key.

- Camera Selection

- Choose the device address need to be controlled

- Pan/Tilt Control

|  | Up: press Down: press Left: press Right: press Back to middle position: press“【HOME】” Press and hold the up/down/left/right key, the pan/tilt movements will keep running, from slow to fast, until it runs to the endpoint; stop as soon as the key is released. |
| --- | --- |

- Set and Clear Presets

|  | Set Preset: press 【SET PRESET】 button, and then press the number key 0-9 to set preset positions. Note: 10 presets via remote control. Call Preset: Press a number key 0-9 directly to call a preset position. Clear Preset: press 【CLEAR PRESET】 button, and then press the number key 0-9 to clear preset positions. Note : press the【#】 key three times continually to clear all presets. |
| --- | --- |

- Zoom Control

|  | ZOOM +: press【ZOOM +】key to zoom in ZOOM - : press 【ZOOM -】key to zoom out Press and hold the keys, the action of focus will keep continue and stop as soon as the key is released. |
| --- | --- |

- Focus Control

- [AUTO]: Enter into auto focus mode - [MANUAL]: Enter into manual focus mode. Press + or - to adjust focus

6. Camera Remote Control Address Setting

|  | 【*】+【＃】+【F1】:Camera Address No.1 【*】+【＃】+【F2】:Camera Address No. 2 【*】+【＃】+【F3】:Camera Address No. 3 【*】+【＃】+【F4】:Camera Address No. 4 |
| --- | --- |

## 3.3 Menu Introduction

### 3.3.1 Main Menu

In normal working mode, press 【MENU】 key to display the menu, using scroll arrow to point at or highlight the selected items.

MENU

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

（Setup）

（Camera）

（P/T/Z）

（Version）

（Restore Default）

[↑↓]Select [← →]Change Value

[Menu]Back[Home]OK

Setup: System parameter setting Camera: Camera parameter setting P/T/Z: Enter into sub menu Version: Enter into sub menu Restore Default: Enter into reset setting, select YES or NO to confirm [↑↓] Select: For selecting menu [← →] Change Value: For modifying parameters [MENU] Back: Press [Menu] to return [Home] OK: Press [Home] to confirm

### 3.3.2 System Setting

Move the pointer to the (Setup) in the Main Menu, click the【HOME】key and enter into the (System Setting) as shown below,

SETUP

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Protocol　　　　　　　   Auto

Visca Address　　　　　　　1

Visca Address Fix　　　   OFF

PELCO-P Address　 1

PELCO-D Address   1

Baudrate　　　　　　　   9600

Auto Filp                 ON

[↑↓]Select　[← →]Change Value

[Menu]Back

[↑↓]选择　　　[← →]修改

[菜单]返回

| Protocol: VISCA/Pelco-P/Pelco-D/Auto Visca Address: VISCA=1~7 Pelco-P=1~255 Pelco-D=1~255 |  |
| --- | --- |
| Baudrate: 2400/4800/9600/115200 Visca Address Fix: On/Off |  |
| Auto Filp: On/Off |  |

### 3.3.3 Camera Setting

Move the pointer to the (CAMERA) in the Main Menu, click the 【HOME】key and enter the (CAMERA) as follow,

CAMERA

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

（Exposure）

（Color）

（Image）

（Focus）

（Noise Reduction）

Style                Default

[↑↓]Select　　[← →]Change Value

[Menu]Back　　 [Home]OK

Exposure: Enter into Exposure setting

Color: Enter into color setting

Image: Enter into image setting

Focus: Enter into focus setting

Noise Reduction: Enter into noise reduction

1）Exposure Setting

Move the pointer to the (EXPOSURE) in the Main Menu, click the【HOME】key and enter into the (Exposure sub menu) as shown below,

EXPOSURE

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Mode　　　　　　　　  Auto

EV　　　　　　　　　  OFF

BLC　　　　　　　　　 OFF

Flicker                 50Hz

G.Limit      3

DRC  2

[↑↓]Select　　[← →]Change Value

[Menu]Back

Mode: Auto, Manual, Shutter priority, Iris priority and Brightness priority.

EV: On/Off (only available in auto mode)

Compensation Level: -7~7 (only available in auto mode when EV is ON)

BLC: ON/OFF for options (only available in auto mode) Dynamic Range: 1~8, close

Anti-Flicker: OFF/50Hz/60Hz for options (only available in Auto/Iris priority/Brightness priority modes)

Gain limit: 0~15 (only available in Auto/ Iris priority /Brightness priority mode)

WDR: Off, 1~8

ShutterPriority:1/25,1/30,1/50,1/60,1/90,1/100,1/120,1/180,1/250,1/350,1/500,1/1000,1/2000,1/3000,1/4000,1/6000, 1/10000 (only available in Manual and Shutter priority mode)

IRIS Priority: OFF, F11.0, F9.6, F8.0, F6.8, F5.6, F4.8, F4.0, F3.4, F2.8, F2.4, F2.0, F1.8(only available in Manual and Iris priority mode)

Brightness: 0~23 (only available in Brightness priority mode) Gain: 0-15 (only available in Auto, and Shutter priority mode)

2）Color

Move the pointer to the (COLOR) in the Main Menu, click the【HOME】and enter the (COLOR sub menu) as follow,

COLOR

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

WB Mode　　　　　　　　Auto

RG Tuning                 0

BG Tuning　　             0

Saturation　　　　　　　   36

Hue   4

AWB Sensitivity  Low

[↑↓]Select　[← →]Change Value

[Menu]Back

| WB Mode: Auto, Manual, One Push, specified temperature color. Saturation: 0-50 available |  |
| --- | --- |
| Red fine-tuning: -10~10 (only available in automatic mode) Blue fine-tunable: -10~10 (only available in automatic mode) |  |
| RED GAIN: 0~255(only available in Manual mode) BLUE GAIN: 0~255(only available in Manual mode) Color Temperature: 2400K-7100K AWB Sensitivity: high/middle/low Hue: 0~8 |  |

3) Image

Move the pointer to the (IMAGE) in the Menu, click the【HOME】and enter into the (IMAGE sub menu) as follow,

IMAGE

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Brightness　　　　　　　　  50

Contrast　　　　　　　　　  50

Sharpness　　　　　　　　   5

B&W-Mode   Color

Gamma   Default

DZoom                    Off

DCI                      Close

Low-Light Mode            OFF

[↑↓]Select　 [← →]Change Value

[Menu]Back

Brightness: 0~100

Contrast: 0~100

Sharpness: 0~16

B&W Mode: color, black/white

DZoom: Off/On

Gamma: Default/0.45/0.50/0.56/0.63

DCI: Dynamic Contrast: Off/1～8

Minimum Illumination: On/Off

4）Focus

Move the pointer to the (FOCUS) in the Menu, click the【HOME】and enter the (FOCUS) as follow,

FOCUS

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Focus Mode   Auto

AF-Zone   All

AF-Sensitivity   Low

[↑↓]Select　　 [← →]Change Value

[Menu]Back

Focus Mode: Auto/manual

AF-Zone: Up/middle/down

AF-Sensitivity: High/middle/low

5）Noise Reduction

Move the pointer to the (NOISE REDUCTION) in the Menu, click the【HOME】and enter the (NOISE REDUCTION) as follow,

NOISE REDUCTION

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

NR-2D2

NR-3D    4

[↑↓]Select　　 [← →]Change Value

[Menu]Back

2D Noise Reduction: Auto, close, 1~7

3D Noise Reduction: Close, 1~8

### 3.3.4 P/T/Z

Move the pointer to the (P/T/Z) in the Main Menu, click the【HOME】and enter the (P/T/Z) as follow,

P/T/Z

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Speed by zoom ON

Zoom speed　　　　　　　　  8

Acc Curve Slow

Preset Speed                 1

[↑↓]Select　 [← →]Change Value [Menu]Back

Depth of Field: Only effective for the remote controller, On/ Off;

(When zooming in, the PT control speed by remoter will become slow)

Zoom Speed: Set the zoom speed for the remote controller, 1~8

Image Freezing: On/Off

Accelerating Curve: Fast/Slow

Preset Speed: This value will not be changed into default due to factory restore

### 3.3.5 Version

Move the pointer to the (VERSION) in the Main Menu, click the【HOME】and enter the (VERSION) as follow,

VERSION

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

MCU Version      3.1.0  2021-10-27

Camera Version    1.0.0  2021-11-15

AF Version        1.0.0  2021-01-12

[↑↓]Select　　 [← →]Change Value

[Menu]Back　　 [Home]OK

MCU Version: Display MCU version information

Camera Version: Display camera version information

AF Version: Display the focus version information

3.3.6 Restore Default

Move the pointer to the (RESTORE DEFAULT) in the Main Menu, click the【HOME】and enter the (RESTORE DEFAULT) as follow,

RESTORE DEFAULT

＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝＝

Restore  Default?      NO

[↑↓]Select　　 [← →]Change Value

[Menu]Back　　 [Home]OK

Restore default: YES/NO. Color style and video format cannot be restored to factory default

Note: If the address of former remoter is not 1 but another one from 2, 3, 4, the corresponding camera address will restore to 1 when all parameters or system parameters are restored. User should change the remoter address to be 1 (press No.1 according to the camera so to get normal operation).

4. Serial Port Communication and Control

The camera could be controlled through RS232/RS485 interface; RS232C serial parameter are as follows:

Baud rate: 2400/4800/9600/115200 bits / sec; Start bit: 1; data bits: 8; Stop bit: 1; Parity: None.

After power on, the camera first goes left, then back to the middle position. Self-test is finished after the zoom moved to the farthest and then back to the nearest position. If the camera saved 0 preset before, it will be back to that position after initialization. At this point, the user can control the camera by the serial commands.

## 4.1 VISCA Protocol list

### 4.1.1 VISCA Protocol Return Command

| Ack/Completion Message |  |  |
| --- | --- | --- |
|  | Command packet | Note |
| ACK | z0 41 FF | Returned when the command is accepted. |
| Completion | z0 51 FF | Returned when the command has been executed. |

z = camera address + 8

| Error Messages |  |  |
| --- | --- | --- |
|  | Command packet | Note |
| Syntax Error | z0 60 02 FF | Returned when the command format is different or when a command with illegal command parameters is accepted |
| Command Not Executable | z0 61 41 FF | Returned when a command cannot be executed due to current conditions. For example, when commands controlling the focus manually are received during auto focus. |

### 4.1.2 VISCA Protocol Control Command

| Command | Function | Command packet | Note |
| --- | --- | --- | --- |
| AddressSet | Broadcast | 88 30 0p FF | p：Address setting |
| IF_Clear | Broadcast | 88 01 00 01 FF | I/F Clear |
| CommandCancel |  | 8x 21 FF |  |
| CAM_Power | On | 8x 01 04 00 02 FF | Power ON/OFF |
|  | Off | 8x 01 04 00 03 FF |  |
| CAM_Zoom | Stop | 8x 01 04 07 00 FF |  |
|  | Tele(Standard) | 8x 01 04 07 02 FF |  |
|  | Wide(Standard) | 8x 01 04 07 03 FF |  |
|  | Tele(Variable) | 8x 01 04 07 2p FF | p = 0(low) - F(high) |
|  | Wide(Variable) | 8x 01 04 07 3p FF |  |
|  | Direct | 8x 01 04 47 0p 0q 0r 0s FF | pqrs: Zoom Position |
| CAM _Focus | Stop | 8x 01 04 08 00 FF |  |
|  | Far(Standard) | 8x 01 04 08 02 FF |  |
|  | Near(Standard) | 8x 01 04 08 03 FF |  |
|  | Far(Variable) | 8x 01 04 08 2p FF | p = 0(low) - F(high) |
|  | Near (Variable) | 8x 01 04 08 3p FF |  |
|  | Direct | 8x 01 04 48 0p 0q 0r 0s FF | pqrs: Focus Position |
|  | Auto Focus | 8x 01 04 38 02 FF |  |
|  | Manual Focus | 8x 01 04 38 03 FF |  |
|  | One Push mode | 8x 01 04 38 04 FF |  |
| CAM _Zoom Focus | Direct | 8x 01 04 47 0p 0q 0r 0s 0t 0u 0v 0w FF | pqrs: Zoom Position tuvw: Focus Position |
| CAM_WB | Auto | 8x 01 04 35 00 FF |  |
|  | 3000K | 8x 01 04 35 01 FF |  |
|  | 4000k | 8x 01 04 35 02 FF |  |
|  | One Push mode | 8x 01 04 35 03 FF |  |
|  | 5000k | 8x 01 04 35 04 FF |  |
|  | Manual | 8x 01 04 35 05 FF |  |
|  | 6500k | 8x 01 04 35 06 FF |  |
|  | 3500K | 8x 01 04 35 07 FF |  |
|  | 4500K | 8x 01 04 35 08 FF |  |
|  | 5500K | 8x 01 04 35 09 FF |  |
|  | 6000K | 8x 01 04 35 0A FF |  |
|  | 7000K | 8x 01 04 35 0B FF |  |
| CAM _RGain | Reset | 8x 01 04 03 00 FF | Manual Control of R Gain |
|  | Up | 8x 01 04 03 02 FF |  |
|  | Down | 8x 01 04 03 03 FF |  |
|  | Direct | 8x 01 04 43 00 00 0p 0q FF | pq: R Gain |
| CAM_ Bgain | Reset | 8x 01 04 04 00 FF | Manual Control of B Gain |
|  | Up | 8x 01 04 04 02 FF |  |
|  | Down | 8x 01 04 04 03 FF |  |
|  | Direct | 8x 01 04 44 00 00 0p 0q FF | pq: B Gain |
| CAM_AE | Full Auto | 8x 01 04 39 00 FF | Automatic Exposure mode |
|  | Manual | 8x 01 04 39 03 FF | Manual Control mode |
|  | Shutter priority | 8x 01 04 39 0A FF | Shutter Priority Automatic Exposure mode |
|  | Iris priority | 8x 01 04 39 0B FF | Iris Priority Automatic Exposure mode |
|  | Bright | 8x 01 04 39 0D FF | Bright mode |
| CAM_Shutter | Reset | 8x 01 04 0A 00 FF | Shutter Setting |
|  | Up | 8x 01 04 0A 02 FF |  |
|  | Down | 8x 01 04 0A 03 FF |  |
|  | Direct | 8x 01 04 4A 00 00 0p 0q FF | pq: Shutter Position |
| CAM_Iris | Reset | 8x 01 04 0B 00 FF | Iris Setting |
|  | Up | 8x 01 04 0B 02 FF |  |
|  | Down | 8x 01 04 0B 03 FF |  |
|  | Direct | 8x 01 04 4B 00 00 0p 0q FF | pq: Iris Position |
| CAM_Gain Limit | Gain Limit | 8x 01 04 2C 0p FF | p: Gain Positon |
| CAM_Bright | Reset | 8x 01 04 0D 00 FF | Bright Setting |
|  | Up | 8x 01 04 0D 02 FF |  |
|  | Down | 8x 01 04 0D 03 FF |  |
|  | Direct | 8x 01 04 4D 00 00 0p 0q FF | pq: Bright Positon |
| CAM_ExpComp | On | 8x 01 04 3E 02 FF | Exposure Compensation ON/OFF |
|  | Off | 8x 01 04 3E 03 FF |  |
|  | Reset | 8x 01 04 0E 00 FF | Exposure Compensation Amount Setting |
|  | Up | 8x 01 04 0E 02 FF |  |
|  | Down | 8x 01 04 0E 03 FF |  |
|  | Direct | 8x 01 04 4E 00 00 0p 0q FF | pq: ExpComp Position |
| CAM_Back Light | On | 8x 01 04 33 02 FF | Back Light Compensation |
|  | Off | 8x 01 04 33 03 FF |  |
| CAM_WDRStrength | Reset | 8x 01 04 21 00 FF | WDR Level Setting |
|  | Up | 8x 01 04 21 02 FF |  |
|  | Down | 8x 01 04 21 03 FF |  |
|  | Direct | 8x 01 04 51 00 00 00 0p FF | p: WDR Level Positon |
| CAM_NR（2D） |  | 8x 01 04 53 0p FF | P=0-7 0:OFF |
| CAM_NR（3D） |  | 8x 01 04 54 0p FF | P=0-8 0:OFF |
| CAM_Gamma |  | 8x 01 04 5B 0p FF | p = 0 – 4 0：Default 1：0.45 2：0.50 3：0.55 4：0.63 |
| CAM_Flicker | OFF | 8x 01 04 23 00 FF | OFF |
|  | 50HZ | 8x 01 04 23 01 FF | 50HZ |
|  | 60HZ | 8x 01 04 23 02 FF | 60HZ |
| CAM_Aperture | Reset | 8x 01 04 02 00 FF | Aperture Control |
|  | Up | 8x 01 04 02 02 FF |  |
|  | Down | 8x 01 04 02 03 FF |  |
|  | Direct | 8x 01 04 42 00 00 0p 0q FF | pq: Aperture Gain |
| CAM_Memory | Reset | 8x 01 04 3F 00 pq FF | pq: Memory Number(=0 to 254) Corresponds to 0 to 9 on the Remote Commander |
|  | Set | 8x 01 04 3F 01 pq FF |  |
|  | Recall | 8x 01 04 3F 02 pq FF |  |
| CAM_LR_Reverse | On | 8x 01 04 61 02 FF | Image Flip Horizontal ON/OFF |
|  | Off | 8x 01 04 61 03 FF |  |
| CAM_PictureFlip | On | 8x 01 04 66 02 FF | Image Flip Vertical ON/OFF |
|  | Off | 8x 01 04 66 03 FF |  |
| CAM_ColorSaturation | Direct | 8x 01 04 49 00 00 00 0p FF | P=0-E 0:60% 1:70% 2:80% 3:90% 4:100% 5:110% 6:120% 7:130% 8:140% 9:150% 10:160% 11:160% 12:180% 13:190% 14:200% |
| CAM_IDWrite |  | 8x 01 04 22 0p 0q 0r 0s FF | pqrs: Camera ID (=0000 to FFFF) |
| SYS_Menu | ON | 8x 01 04 06 06 02 FF | Turn on the menu screen |
|  | OFF | 8x 01 04 06 06 03 FF | Turn off the menu screen |
| IR_Receive | ON | 8x 01 06 08 02 FF | IR(remote commander)receive On/Off |
|  | OFF | 8x 01 06 08 03 FF |  |
| IR_ReceiveReturn | On | 8x 01 7D 01 03 00 00 FF | IR(remote commander)receive message via the VISCA communication ON/OFF |
|  | Off | 8x 01 7D 01 13 00 00 FF |  |
| CAM_SettingReset | Reset | 8x 01 04 A0 10 FF | Reset Factory Setting |
| CAM_Brightness | Direct | 8x 01 04 A1 00 00 0p 0q FF | pq: Brightness Position |
| CAM_Contrast | Direct | 8x 01 04 A2 00 00 0p 0q FF | pq: Contrast Position |
| CAM_Flip | OFF | 8x 01 04 A4 00 FF | Single Command For Video Flip |
|  | Flip-H | 8x 01 04 A4 01 FF |  |
|  | Flip-V | 8x 01 04 A4 02 FF |  |
|  | Flip-HV | 8x 01 04 A4 03 FF |  |
| CAM_VideoSystem | Set camera video system | 8x 01 06 35 00 0p FF | P: 0~E Video format 0:1080P60 8:720P30 1:1080P50 9:720P25 2:1080i60 A：1080P59.94 3:1080i50 B：1080i59.94 4:720P60 C：720P59.94 5:720P50 D：1080P29.97 6:1080P30 E：720P29.97 7:1080P25 |
| Pan_tiltDrive | Up | 8x 01 06 01 VV WW 03 01 FF | VV: Pan speed 0x01 (low speed) to 0x18 (high speed) WW: Tilt speed 0x01 (low speed) to 0x14 (high speed) YYYY: Pan Position ZZZZ: Tilt Position |
|  | Down | 8x 01 06 01 VV WW 03 02 FF |  |
|  | Left | 8x 01 06 01 VV WW 01 03 FF |  |
|  | Right | 8x 01 06 01 VV WW 02 03 FF |  |
|  | Upleft | 8x 01 06 01 VV WW 01 01 FF |  |
|  | Upright | 8x 01 06 01 VV WW 02 01 FF |  |
|  | DownLeft | 8x 01 06 01 VV WW 01 02 FF |  |
|  | DownRight | 8x 01 06 01 VV WW 02 02 FF |  |
|  | Stop | 8x 01 06 01 VV WW 03 03 FF |  |
|  | AbsolutePosition | 8x 01 06 02 VV WW 0Y 0Y 0Y 0Y 0Z 0Z 0Z 0Z FF |  |
|  | RelativePosition | 8x 01 06 03 VV WW 0Y 0Y 0Y 0Y 0Z 0Z 0Z 0Z FF |  |
|  | Home | 8x 01 06 04 FF |  |
|  | Reset | 8x 01 06 05 FF |  |
| Pan-tiltLimitSet | Set | 8x 01 06 07 00 0W 0Y 0Y 0Y 0Y 0Z 0Z 0Z 0Z FF | W:1 UpRight 0:DownLeft YYYY: Pan Limit Position(TBD) ZZZZ: Tilt Limit Position(TBD) |
|  | Clear | 8x 01 06 07 01 0W 07 0F 0F 0F 07 0F 0F 0F FF |  |

### 4.1.3 VISCA Protocol Inquiry Command

| Command | Command Packet | Return Packet | Note |
| --- | --- | --- | --- |
| CAM_PowerInq | 8x 09 04 00 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off(Standby) |
| CAM_ZoomPosInq | 8x 09 04 47 FF | y0 50 0p 0q 0r 0s FF | pqrs: Zoom Position |
| CAM_FocusAFModeInq | 8x 09 04 38 FF | y0 50 02 FF | Auto Focus |
|  |  | y0 50 03 FF | Manual Focus |
|  |  | y0 50 04 FF | One Push mode |
| CAM_FocusPosInq | 8x 09 04 48 FF | y0 50 0p 0q 0r 0s FF | pqrs: Focus Position |
| CAM_WBModeInq | 8x 09 04 35 FF | y0 50 00 FF | Auto |
|  |  | y0 50 01 FF | 3000K |
|  |  | y0 50 02 FF | 4000K |
|  |  | y0 50 03 FF | One Push Mode |
|  |  | y0 50 04 FF | 5000K |
|  |  | y0 50 05 FF | Manual |
|  |  | y0 50 00 FF | 6500K |
|  |  | y0 50 06 FF | 6500K |
|  |  | y0 50 07 FF | 3500K |
|  |  | y0 50 08 FF | 4500K |
|  |  | y0 50 09 FF | 5500K |
|  |  | y0 50 0A FF | 6000K |
| CAM_RGainInq | 8x 09 04 43 FF | y0 50 0B FF | 7000K |
| CAM_BGainInq | 8x 09 04 44 FF | y0 50 00 00 0p 0q FF | pq: B Gain |
| CAM_AEModeInq | 8x 09 04 39 FF | y0 50 00 FF | Full Auto |
|  |  | y0 50 03 FF | Manual |
|  |  | y0 50 0A FF | Shutter priority |
|  |  | y0 50 0B FF | Iris priority |
|  |  | y0 50 0D FF | Bright |
| CAM_ShutterPosInq | 8x 09 04 4A FF | y0 50 00 00 0p 0q FF | pq: Shutter Position |
| CAM_IrisPosInq | 8x 09 04 4B FF | y0 50 00 00 0p 0q FF | pq: Iris Position |
| CAM_Gain LimitInq | 8x 09 04 2C FF | y0 50 0p FF | p: Gain Positon |
| CAM_ BrightPosiInq | 8x 09 04 4D FF | y0 50 00 00 0p 0q FF | pq: Bright Position |
| CAM_ExpCompModeInq | 8x 09 04 3E FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_ExpCompPosInq | 8x 09 04 4E FF | y0 50 00 00 0p 0q FF | pq: ExpComp Position |
| CAM_BacklightModeInq | 8x 09 04 33 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_WDRStrengthInq | 8x 09 04 51 FF | y0 50 00 00 00 0p FF | p: WDR Strength |
| CAM_NRLevel(2D) Inq | 8x 09 04 53 FF | y0 50 0p FF | P: 2DNRLevel |
| CAM_NRLevel(3D) Inq | 8x 09 04 54 FF | y0 50 0p FF | P:3D NRLevel |
| CAM_FlickerModeInq | 8x 09 04 55 FF | y0 50 0p FF | p: Flicker Settings(0: OFF, 1: 50Hz, 2:60Hz) |
| CAM_ApertureInq | 8x 09 04 42 FF | y0 50 00 00 0p 0q FF | pq: Aperture Gain |
| CAM_PictureEffectModeInq | 8x 09 04 63 FF | y0 50 00 FF | Off |
|  |  | y0 50 04 FF | B&W |
| CAM_MemoryInq | 8x 09 04 3F FF | y0 50 0p FF | p: Memory number last operated. |
| SYS_MenuModeInq | 8x 09 06 06 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_LR_ReverseInq | 8x 09 04 61 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_PictureFlipInq | 8x 09 04 66 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_ColorSaturationInq | 8x 09 04 49 FF | y0 50 00 00 00 0p FF | p: Color Gain setting 0h (60%) to Eh (130%) |
| CAM_IDInq | 8x 09 04 22 FF | y0 50 0p FF | p: Gamma ID |
| IR_ReceiveInq | 8x 09 06 08 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| IR_ReceiveReturn |  | y0 07 7D 01 04 00 FF | Power ON/OFF |
|  |  | y0 07 7D 01 04 07 FF | Zoom tele/wide |
|  |  | y0 07 7D 01 04 38 FF | AF ON/OFF |
|  |  | y0 07 7D 01 04 33 FF | Camera _Backlight |
|  |  | y0 07 7D 01 04 3F FF | Camera _Memery |
|  |  | y0 07 7D 01 06 01 FF | Pan_titleDriver |
| CAM_BrightnessInq | 8x 09 04 A1 FF | y0 50 00 00 0p 0q FF | pq: Brightness Position |
| CAM_ContrastInq | 8x 09 04 A2 FF | y0 50 00 00 0p 0q FF | pq: Contrast Position |
| CAM_FlipInq | 8x 09 04 A4 FF | y0 50 00 FF | Off |
|  |  | y0 50 01 FF | Flip-H |
|  |  | y0 50 02 FF | Flip-V |
|  |  | y0 50 03 FF | Flip-HV |
| CAM_GammaInq | 8x 09 04 5B FF | y0 50 0p FF | p: Gamma setting |
| CAM_VersionInq | 8x 09 00 02 FF | y0 50 ab cd mn pq rs tu vw FF | ab cd : vender ID ( 0220 ) mn pq : model ID ST ( 0510 ) 、 U2( 0512 )、U3 ( 0513 ) rs tu : ARM Version vw : reserve |
| VideoSystemInq | 8x 09 06 23 FF | y0 50 0p FF | P: 0~E Video format 0:1080P60 8:720P30 1:1080P50 9:720P25 2:1080i60 A：1080P59.94 3:1080i50 B：1080i59.94 4:720P60 C：720P59.94 5:720P50 D：1080P29.97 6:1080P30 E：720P29.97 7:1080P25 |
| Pan-tiltMaxSpeedInq | 8x 09 06 11 FF | y0 50 ww zz FF | ww: Pan Max Speed zz: Tilt Max Speed |
| Pan-tiltPosInq | 8x 09 06 12 FF | y0 50 0w 0w 0w 0w 0z 0z 0z 0z FF | wwww: Pan Position zzzz: Tilt Position |

Note:[X] in the above table indicates the camera address to be operated,【y】=【x + 8】.

## 4.2 Pelco-D Protocol Command List

| Function | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Up | 0xFF | Address | 0x00 | 0x08 | Pan Speed | Tilt Speed | SUM |
| Down | 0xFF | Address | 0x00 | 0x10 | Pan Speed | Tilt Speed | SUM |
| Left | 0xFF | Address | 0x00 | 0x04 | Pan Speed | Tilt Speed | SUM |
| Right | 0xFF | Address | 0x00 | 0x02 | Pan Speed | Tilt Speed | SUM |
| Upleft | 0xFF | Address | 0x00 | 0x0C | Pan Speed | Tilt Speed | SUM |
| Upright | 0xFF | Address | 0x00 | 0x0A | Pan Speed | Tilt Speed | SUM |
| DownLeft | 0xFF | Address | 0x00 | 0x14 | Pan Speed | Tilt Speed | SUM |
| DownRight | 0xFF | Address | 0x00 | 0x12 | Pan Speed | Tilt Speed | SUM |
| Zoom In | 0xFF | Address | 0x00 | 0x20 | 0x00 | 0x00 | SUM |
| Zoom Out | 0xFF | Address | 0x00 | 0x40 | 0x00 | 0x00 | SUM |
| Focus Far | 0xFF | Address | 0x00 | 0x80 | 0x00 | 0x00 | SUM |
| Focus Near | 0xFF | Address | 0x01 | 0x00 | 0x00 | 0x00 | SUM |
| Stop | 0xFF | Address | 0x00 | 0x00 | 0x00 | 0x00 | SUM |
| Set Preset | 0xFF | Address | 0x00 | 0x03 | 0x00 | Preset ID | SUM |
| Clear Preset | 0xFF | Address | 0x00 | 0x05 | 0x00 | Preset ID | SUM |
| Call Preset | 0xFF | Address | 0x00 | 0x07 | 0x00 | Preset ID | SUM |
| Query Pan Position | 0xFF | Address | 0x00 | 0x51 | 0x00 | 0x00 | SUM |
| Query Pan Position Response | 0xFF | Address | 0x00 | 0x59 | Value High Byte | Value Low Byte | SUM |
| Query Tilt Position | 0xFF | Address | 0x00 | 0x53 | 0x00 | 0x00 | SUM |
| Query Tilt Position Response | 0xFF | Address | 0x00 | 0x5B | Value High Byte | Value Low Byte | SUM |
| Query Zoom Position | 0xFF | Address | 0x00 | 0x55 | 0x00 | 0x00 | SUM |
| Query Zoom Position Response | 0xFF | Address | 0x00 | 0x5D | Value High Byte | Value Low Byte | SUM |

## 4.3 Pelco-P Protocol Command List

| Function | Byte1 | Byte2 | Byte3 | Byte4 | Byte5 | Byte6 | Byte7 | Byte8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Up | 0xA0 | Address | 0x00 | 0x08 | Pan Speed | Tilt Speed | 0xAF | XOR |
| Down | 0xA0 | Address | 0x00 | 0x10 | Pan Speed | Tilt Speed | 0xAF | XOR |
| Left | 0xA0 | Address | 0x00 | 0x04 | Pan Speed | Tilt Speed | 0xAF | XOR |
| Right | 0xA0 | Address | 0x00 | 0x02 | Pan Speed | Tilt Speed | 0xAF | XOR |
| Upleft | 0xA0 | Address | 0x00 | 0x0C | Pan Speed | Tilt Speed | 0xAF | XOR |
| Upright | 0xA0 | Address | 0x00 | 0x0A | Pan Speed | Tilt Speed | 0xAF | XOR |
| DownLeft | 0xA0 | Address | 0x00 | 0x14 | Pan Speed | Tilt Speed | 0xAF | XOR |
| DownRight | 0xA0 | Address | 0x00 | 0x12 | Pan Speed | Tilt Speed | 0xAF | XOR |
| Zoom In | 0xA0 | Address | 0x00 | 0x20 | 0x00 | 0x00 | 0xAF | XOR |
| Zoom Out | 0xA0 | Address | 0x00 | 0x40 | 0x00 | 0x00 | 0xAF | XOR |
| Stop | 0xA0 | Address | 0x00 | 0x00 | 0x00 | 0x00 | 0xAF | XOR |
| Focus Far | 0xA0 | Address | 0x01 | 0x00 | 0x00 | 0x00 | 0xAF | XOR |
| Focus Near | 0xA0 | Address | 0x02 | 0x00 | 0x00 | 0x00 | 0xAF | XOR |
| Set Preset | 0xA0 | Address | 0x00 | 0x03 | 0x00 | Preset ID | 0xAF | XOR |
| Clear Preset | 0xA0 | Address | 0x00 | 0x05 | 0x00 | Preset ID | 0xAF | XOR |
| Call Preset | 0xA0 | Address | 0x00 | 0x07 | 0x00 | Preset ID | 0xAF | XOR |
| Query Pan Position | 0xA0 | Address | 0x00 | 0x51 | 0x00 | 0x00 | 0xAF | XOR |
| Query Pan Position Response | 0xA0 | Address | 0x00 | 0x59 | Value High Byte | Value Low Byte | 0xAF | XOR |
| Query Tilt Position | 0xA0 | Address | 0x00 | 0x53 | 0x00 | 0x00 | 0xAF | XOR |
| Query Tilt Position Response | 0xA0 | Address | 0x00 | 0x5B | Value High Byte | Value Low Byte | 0xAF | XOR |
| Query Zoom Position | 0xA0 | Address | 0x00 | 0x55 | 0x00 | 0x00 | 0xAF | XOR |
| Query Zoom Position Response | 0xA0 | Address | 0x00 | 0x5D | Value High Byte | Value Low Byte | 0xAF | XOR |

# 5. Maintenance and Troubleshooting

## 5.1 Camera Maintenance

1) Please power off the camera and disconnect the power adapter and socket, if it’s not used for a long run.

2) Use soft cloth or tissue to clean the camera cover.

3) Wipe it with a soft, dry cloth when cleaning the camera lens. Wipe it gently with a mild detergent if needed. Do not use strong or corrosive detergents to avoid scratching the lens and affecting the video quality.

## 5.2 Troubleshooting

- No video output

a. Check whether the camera power supply is connected, the voltage is normal, the power indicator is lit.

b. Whether the machine could do self-check after restarted.

c. Check whether the bottom of the DIP switch is the normal operating mode (see Table 2.2 and Table 2.3)

d. Check whether the video output cable or video display is normal

2) No image sometimes

a. Check whether the video output cable or video display is normal

3) Video dithering when zoom-in or zoom-out

a. Check whether the camera installation position is solid

b. Whether there is shaking machine or objects around the camera

4) Remote control not works

a. Remote control address is set to 1 (if the machine is set back to the factory defaults, remote control addresses need to be back to 1 too)

b. Check whether the battery is installed on the remote controller or low.

c, Check the camera working mode is the normal operating mode (see Table 2.2 and Table 2.3)

d. Check the menu whether is closed, camera control through remote controller is only available after exiting the menu. If video output from LAN, menu will not be displayed, menu will automatically exists 30s later, and then it can be controlled by remote controller.

- Serial port not works

a. Check whether the camera serial device protocol, baud rate, address is consistent

b. Check whether the control cable is connected properly

c. Check whether the camera working mode is the normal operating mode (see Table 2.2 and Table 2.3)

# 6. Copyright Statement

All the contents in this manual and its copyright are owned by the company. No one is allowed to imitate, copy, or translate this manual without the company’s permission. This manual contains no guarantee, standpoint expression or other implies in any form. Product specification and information in this manual is for reference only and subject to change without notice.

All rights reserved. No reproducing is allowed without acknowledgement.